For Battery Optimization:

Phone Setup

  - Fully charge phone and leave on charger

  - Confirm phone is factory reset, if not, reset to factory settings

  - Setup –

    - English/United States

    - Setup up manually

    - Choose WiFI network

    - Enter password

    - Don’t transfer apps and data

    - Apple ID for site and password/enter verification code which will be sent to the phone number on the Apple ID

  - Enable location services, apple pay no, Keychain off, Siri off, screen time off, don’t share analytics, light theme

  - Update phone OS if needed

  - Auto update of OS turn off

  - Check battery health % on phone and watch, must be >85% on watch, phone >50%

  - Delete all apps where possible or hide except camera, app store, settings, watch

Watch Setup

  - Fully charge watch and leave on charger

  - Pair watch to phone

  - Configure -Left wrist, route tracking off, watch passcode none, activity off, Siri off

  - Check watch battery health - should be >90%

  - Update watch OS

  - Auto OS update off on watch

  - App view to “list” and delete all apps possible except leave Camera, App Store, Health

  - Choose watch face

  - Add battery charge level to watch face as a complication. (This is done on the watch.)

  - Set for maximum battery life

    - Settings/General → Background app refresh → Disable

    - Settings/Display and Brightness  → Disable all options (“Wakes on Wrist Raise”, “Wake Screen on Crown Up”, “Auto Launch Audio apps”)

    - Settings/Display and Brightness → Wake Duration → Wake for 15 seconds

    - Display & Brightness (Minimal Brightness, “Always on” → Disable) (Doesn’t Apply to SE)

    - Settings/Siri → Disable all options

    - Settings/Privacy and Security/Microphone/ “Measure Sounds” off

    - Settings/Noise/ “Environmental Sound Measurements” off

    - Settings/Activity → All options off

    - Workout → Disable all options